A Big Earth Data Platform for Three Poles

WATER: Dataset of airborne WiDAS mission in the A'rou flight zone on Jul. 7, 2008

1、Description

The dataset of airborne WiDAS mission was obtained in the A'rou flight zone on Jul. 7, 2008. Due to cloud/cloud shadow influence, atmospheric correction could not be performed, and geometric registration was performed manually instead of automatic matching. Level-2B (after radiometric and manual geometric corrections) and mosaic images were available for users. For the visible near infrared band the resolution is 1.25m, Radiance was recorded (W/ (sr•m^2•nm);DN=Radiance*100000); for TIR band, the brightness temperature was recorded (℃; DN=Brightness_Temperature*100) .
     The flying time of each route was as follows:

{|
! id
! flight
! relative height
! starttime
! endtime
! data size
! data state
! data quality
! ground targets 
|-
| 1 || 6#1 || 1500m || 13:43:18 || 13:46:26 || 48  || incomplete || incomplete
|-
| 2 || 6#3 || 1500m || 13:52:26 || 13:55:18 || 43  || incomplete || incomplete
|-
| 3 || 6#5 || 1500m || 13:59:30 || 14:02:38 || 48  || incomplete || incomplete || A’rou freeze/thaw observation station
|-
| 4 || 6#7 || 1500m || 14:08:02 || 14:11:02 || 46  || incomplete || incomplete
|}

2、Keywords

Theme：Thermal imager,Remote Sensing Technology,Wide-angle infrared dual-mode line/Area array scanner,CCD
Discipline：Remote Sensing Technology
Places：Heihe River Basin, Arid Region Hydrology in the Middle Reaches, A'rou flight zone
Time：

3、Data details

1.Scale：None

2.Projection：4326

3.Filesize：530.2MB

4.Data format：

4、Space scope

| - | north：38.15 | - |
| --- | --- | --- |
| west：100.15 | - | east：100.65 |
| - | south：38.0 | - |

5、Time frame:2008-07-16 18:43:00+00:00--2008-07-16 23:45:00+00:00

6、Reference method

References to data:

MA Mingguo, WANG Heshun. WATER: Dataset of airborne WiDAS mission in the A'rou flight zone on Jul. 7, 2008. A Big Earth Data Platform for Three Poles, doi:10.3972/water973.0216.db2012

References to articles:

方莉, 刘强, 肖青, 柳钦火, 刘志刚. 黑河试验中机载红外广角双模式成像仪的设计及实现. 地球科学进展, 2009, 24(7): 696-704.

刘强, 肖青, 刘志刚, 方莉, 彭菁菁, 李波. 黑河综合遥感联合试验中机载WIDAS数据的预处理方法. 遥感技术与应用, 2010, 25(6): 797-804.

7、Supporting project information

The CAS (Chinese Academy of Sciences) Action Plan for West Development Project
National Program on Key Basic Research Project (973 Program

8、Data resource provider

name: WANG Heshun
unit: 
email: heshun8336@163.com

name: MA Mingguo
unit: Cold and Arid Regions Environmental and Engineering Research Institute, Chinese Academy of Sciences
email: mmg@lzb.ac.cn